# NATIONAL ANNEX

# TO

# CYS EN 1997-2:2024 Eurocode 7: Geotechnical design

# Part 2: Ground properties

# For Public Enquiry

12-Jan-26

Prepared by: CYS TC 18 EUROCODES

Cyprus Organisation for Standardisation (CYS)

INTRODUCTION

This National Annex has been prepared by the CYS TC 18: Eurocodes National Standardisation Technical Committee of Cyprus Organisation for Standardisation. (CYS)

# SCOPE

This National Annex is to be used together with CYS EN 1997-2:2024

This National Annex gives:

Nationally determined parameters for the following clauses of CYS EN 1997-2:2024 where National choice is allowed (see Section NA 2)

H.3 (1)

H.3 (2)

Decisions on the use of Informative Annexes A, B, C, D, E,F,G,H (see Section NA 3)

References to non-contradictory complementary information to assist the user to apply CYS EN 1997-2:2004. In this National Annex such information is provided for the following clauses in CYS EN 1997-2:2024 (see Section NA 4)

# NATIONALLY DETERMINED PARAMETERS

Clause H.3 (1) Spacing and number of investigation locations

Table H.1 (CYS) provides Values of Xmax for structures in Geotechnical Category 2.

## Clause H.3 (2) Spacing and number of investigation locations

Table H.1 (CYS) provides Values of Nmin for structures in Geotechnical Category 2.

# Table H.1(CYS) — Maximum spacing and minimum number of investigation locations for structures in Geotechnical Category 2

| Structures |  | Maximum spacing Xmax | Minimum number Nmin |
| --- | --- | --- | --- |
| Low-rise structures |  | 30 m | 3 |
| High-rise structures | 4-10 storeys | 25 m | 3-4b |
|  | 11-20 storeys | 20 m | 3-5b |
|  | > 20 storeys | 15 m | 3-6b |
| Estate roads, parking areas and pavements |  | 40 m | 2 |
| Silos and tanks |  | 15 m | 3 |
| Bridges piers and abutments |  | 1 per pier/base |  |
| Power lines |  | 1 per pylon |  |
| Wind turbines |  | 2 per turbine |  |
| Retaining structures |  | 150 m | -a |
| Slopes and cuttings | < 3 m high | 100 m | -a |
|  | ≥ 3 m high | 50 m | -a |
| Embankments and reinforced fill structures | < 3 m high | 200 m | -a |
|  | ≥ 3 m high | 100 m | -a |
| Excavations in urban areas > 5 m deep from ground surface |  | 25 m | 3 |
| a Where no spacing or number of locations is given, it is assessed on a project-specific basis. b Underlined numbers are more appropriate for complex structures. |  |  |  |

# DECISION ON USE OF THE INFORMATIVE ANNEXES A,B,C,D,E,F,G,H

## Annex A

Annex A may be used

## Annex B

Annex B may be used

## Annex C

Annex C may be used

## Annex D

Annex D may be used

## Annex E

Annex E may be used

## Annex F

Annex F may be used

## Annex G

Annex G may be used

## Annex H

Annex H may be used

# REFERENCES TO NON-CONTRADICTORY COMPLEMENTARY INFORMATION

None